# what would be a predicted psi when a foley cath is inserted in a normal healthy patient

If you mean bladder (intravesical) pressure measured through a Foley catheter in a normal, healthy, non-distended bladder, a typical resting value is roughly 5–15 cmH2O (about 4–11 mmHg). It rises normally with ﬁlling and can transiently exceed that during coughing, straining, or voiding.

If you mean intra-abdominal pressure (IAP) measured via the Foley catheter, the usual normal range is about 0–7 mmHg (roughly 5–9 cmH2O); values above 12 mmHg deﬁne intra-abdominal hypertension, and above 20 mmHg with organ dysfunction suggests abdominal compartment syndrome.

# Important distinction

A Foley catheter itself does not create or “have” a PSI. It drains urine, and pressure depends on:

Whether the bladder is empty, normally ﬁlled, or overdistended

Abdominal muscle tone, obesity, coughing, or straining

Whether the tubing/system is open to drainage or clamped for measurement

The measurement technique (transducer zeroed at the pubic symphysis, patient supine, etc.)

For a healthy patient with an unclamped Foley draining freely, the pressure at the catheter tip is generally very low—o en near zero relative to atmosphere—because urine drains rather than accumulating.

# Practical interpretation

| Measurement | Typical normal range |
| --- | --- |
| Resting intravesical/bladder pressure | ~5–15 cmH2O |
| Intra-abdominal pressure via Foley | ~0–7 mmHg |
| Foley draining freely | O en near atmospheric / very low pressure |

A Foley is commonly used for drainage and urine-output monitoring; the retention balloon is typically inﬂated with sterile water a er conﬁrming placement, usually about 10 mL for a standard adult catheter unless the device speciﬁes otherwise.[1] [2]

If youʼre asking because you saw a pressure reading on a monitor or urodynamic study, tell me the units and whether it was bladder pressure, abdominal pressure, or irrigation pressure—I

can help interpret it.

⁂

- https://www.ncbi.nlm.nih.gov/books/NBK539839/

- https://cdn.dal.ca/content/dam/dalhousie/pdf/faculty/medicine/departments/department-sites/urology/ClinicalClerks   hipBook/Foley_POM.pdf

- https://www.careoxy.com/blog/foley-catheterisation

- https://www.brooklynentdoc.com/patient-education/health-library?DOCHWID=zm2412

- http://cusbed.cumhuriyet.edu.tr/tr/download/article-file/2097635

- https://www.aidstat.com/foley_catheter_size_calculator

- https://www.yinuomed.net/news/pvc-vs-silicone-foley-catheter-comparison.html

- https://www.cliﬀsnotes.com/study-notes/27702850

- https://www.scribd.com/document/537779656/LECTURETTE-Foley -catheter

- https://bepls.com/spl_(2)_2023/63.pdf