# 700ml over 5-10 seconds time span into mmhg of pressure

cath was FR20 3-way silicone ﬂuid viscosity was like water.

bladder draining like a ﬂuid cannon on cath insertion and cath length 55cm

Using Poiseuille ﬂow through the catheter, draining 700 mL in 5–10 seconds implies an average bladder pressure of roughly 25–90 mmHg, depending mostly on the catheterʼs actual internal drainage-lumen diameter.

# Estimate

Assuming water-like viscosity, a 55 cm catheter, and laminar ﬂow:

| Assumed drainage-lumen ID | 700 mL in 5 s | 700 mL in 10 s |
| --- | --- | --- |
| 4.7 mm (large lumen) | ~48 mmHg | ~24 mmHg |
| 4.0 mm | ~92 mmHg | ~46 mmHg |
| 3.5 mm (more typical for a 3-way catheter) | ~157 mmHg | ~78 mmHg |

A 20 Fr catheter is about 6.7 mm outer diameter, but a 3-way silicone Foley has a smaller drainage channel because the same tube also carries irrigation and balloon channels. A reasonable working estimate for the drainage lumen is around 3.5–4.0 mm ID, so the most plausible average pressure is about 50–90 mmHg, with ~80 mmHg a reasonable midpoint if it drained in about 10 seconds.

# What that means

That is far above the normal resting bladder pressure of about 5–15 cmH2O (roughly 4–11 mmHg).

It is consistent with a very full, high-pressure bladder with strong detrusor contraction, not ordinary passive drainage.

The “ﬂuid cannon” appearance is expected when a catheter is inserted into an overdistended bladder: the pressure head plus bladder contraction drives urine rapidly through the tube.

The calculation is only an approximation: turbulence, catheter entrance/exit losses, eyelets, kinking, and the fact that pressure falls as the bladder empties all matter. Actual

peak pressure was likely higher than the average calculated here.

# Safety note

Rapid decompression of a very full bladder can cause bladder-wall bleeding, hematuria, hypotension, or vagal symptoms. In practice, especially with a large residual volume, drainage is o en initially controlled or intermittent rather than allowing a full 700 mL to evacuate in seconds. If this happened to a person and there is hematuria, dizziness, pain, or inability to urinate a erward, they should seek urgent medical assessment.

⁂

- https://pubmed.ncbi.nlm.nih.gov/12576831/

- https://repub.eur.nl/pub/14767/3798616.pdf

- https://reference.medscape.com/cc2/p10/neuro-urology -eau-guidelines-2026a10012bv

- https://admin-api.polscientific.com/index.php/bladder/article/download/826/pdf_7/1275

- https://abdominalkey.com/the-normal-study/

- https://www.academia.edu/92025776/Comparison_of_water_and_air_charged_transducer_catheter_pressures_in_th e_evaluation_of_cystometrogram_and_voiding_pressure_studies

- https://quizlet.com/study-guides/basic-concepts-of-mechanical-ventilation-7c76f50c-f87d-4d02-ad41-b4e4e98b394c

- https://pubmed.ncbi.nlm.nih.gov/12707877/

- https://pubmed.ncbi.nlm.nih.gov/10573772/

- https://pubmed.ncbi.nlm.nih.gov/7087025/

- https://pubmed.ncbi.nlm.nih.gov/4897285/

- https://pubmed.ncbi.nlm.nih.gov/22190105/

- https://pmc.ncbi.nlm.nih.gov/articles/PMC3726771/

- https://www.ncbi.nlm.nih.gov/books/NBK562310/

- https://www.ics.org/Abstracts/Publish/44/000104.pdf